Helveticore OÜ

Technical Report & Prototype Manual

Strategic IoT & Edge Computing Portfolio — implementation status

Document: Technical Report & Prototype Manual (TRM)

Scope: EdgeStream-GW — what has been implemented so far + how to use the prototype

Prepared for: Helveticore OÜ — Tallinn, Estonia

Date: 2026-10-05

Methodology: CMMI-DEV v2.0, Level 3 (Defined) — V-Model / TDD / CBSE

Status: In progress — implementation milestone report

# 1. Executive summary

This report records the current state of implementation of the Helveticore strategic IoT and Edge Computing portfolio, with a focus on the EdgeStream-GW project — the only project that has reached working, test-verified code to date. It recalls the core engineering concepts that govern the portfolio, details exactly what has been built, and provides a hands-on manual for running and extending the prototype.

Two of the three portfolio projects (Fog-Crypto Core and NetSentry-IoT) are at the specification stage; no executable code exists for them yet. EdgeStream-GW has three implemented, test-first software components: the Protocol Adapters (Modbus RTU and Modbus TCP normalization into the canonical Frame object), the Edge Analytics PS aggregation library, and a deterministic 50-meter virtual test bench. All 66 automated tests pass, with the adapter implementations at 100% branch coverage and the Edge Analytics PS module at 100% statement coverage.

# 2. Core concepts (recall)

## 2.1 Methodology — CMMI Level 3 (Defined)

Every portfolio project follows the same engineering rigor, governed by a CMMI-DEV v2.0 Level 3 (Defined) framework expressed through three complementary practices:

Macro-architecture — top-down design and bottom-up validation/verification (V-Model).

Micro-architecture (TDD) — proprietary components are built test-first with mocked inputs.

Integration (CBSE) — qualify and reuse proven open-source components; build in-house whatever requires strict control (Fail-Fast parsing, strict schemas).

The framework maps CMMI practice areas to concrete repository artifacts: requirements (REQ.md + RTM.md), design (SDD.md + decision-log.md), testbench/ (virtual verification), performance/ (KPI measurement), and src/ (technical sub-projects). The Build-vs-Reuse rule is explicit: reuse when footprint, latency and protocol compliance are proven; build in-house when Fail-Fast behavior, a proprietary schema, or a CMMI L3 quality guarantee is required.

## 2.2 The strategic triptych (2026–2027)

| Project | Positioning | Impact KPIs | Status |
| --- | --- | --- | --- |
| EdgeStream-GW | IoT Gateway & Edge Analytics (Smart Grid) | −75% data volume to Cloud; local actuation < 50 ms | Code implemented (this report) |
| Fog-Crypto Core | FPGA hardware accelerator (HW/SW co-design, security) | gateway CPU load ÷ 10; line-rate encryption | Specification only |
| NetSentry-IoT | Active network cybersecurity probe (NIS2 / GDPR) | malicious packet detection & isolation < 5 ms; no Cloud signature dependency | Specification only |

## 2.3 EdgeStream-GW architecture

EdgeStream-GW is an ultra-lightweight industrial software gateway positioned between physical perception nodes (sensors, meters, PLCs) and the Cloud, targeting the Puzhi ZU2CG / ZU3EG (Zynq UltraScale+) with a PS/PL (software / FPGA fabric) partitioning. Its pipeline decouples collection, normalization, analytics and publication:

south-drivers  ->  protocol-adapters  ->  edge-analytics  ->  ingestion-core  ->  northbound-publishers

(PS filtering / PL aggregation)          (async broker)      (MQTT / OPC UA)

|

+-> local-action-engine  (fast path, < 50 ms)

+-> store-and-forward    (offline SQLite safety queue)

The PS (software) side hosts asynchronous services (Docker, asyncio, MQTT/OPC UA); the PL (FPGA) side hosts deterministic filtering, FFT and low-latency triggers, connected through the AXI bus and interrupts. The two headline KPIs — −75% cloud volume and < 50 ms actuation — are realized respectively by Edge Analytics (REQ-001a/001b) and the Local Action Engine (REQ-003).

## 2.4 Key definitions

Frame — the standardized internal JSON object representing parsed telemetry.

ActionCommand — the strict structure of an immediate hardware write command (e.g. Modbus FC 05).

Fail-Fast — immediate, explicit rejection of corrupted data without silent repair.

CorruptedPayloadError — the proprietary exception raised by the Protocol Adapters on any malformed payload.

Edge Analytics — the logical PS/PL construct performing data reduction (the −75% KPI).

AXI Bus — the internal high-bandwidth bus between PS and PL.

Sparkplug B — the industrial MQTT specification used northbound (namespace, states, Protobuf).

# 3. What has been implemented so far

## 3.1 Portfolio scaffold

The CMMI Level 3 scaffold is in place for all three projects: the shared methodology charter (process/cmmi-charter.md), the template library (process/templates/), and the mandatory per-project artifacts (requirements/, design/, testbench/, performance/, src/). Source documents (portfolio strategy, the EdgeStream-GW SDD IEEE 1016, company register) are preserved immutably under docs/.

## 3.2 Protocol Adapters (built in-house, TDD)

The first implemented module, chosen by Decision 5 (pure, deterministic, no external dependencies). It translates raw binary protocol payloads into the canonical Frame object, applying Fail-Fast: any corrupted or incomplete payload raises CorruptedPayloadError and is discarded — never silently repaired (REQ-004, Decision 2).

Frame / Source / Measurement — strict Pydantic v2 models with extra="forbid", exact types and bounded ranges (frame.py).

RawPayload — the frozen input contract handed by a South Driver (base.py).

ProtocolAdapter (ABC) — the abstract parse() contract every adapter implements.

CorruptedPayloadError — proprietary exception carrying a reason_code (errors.py).

ModbusRtuAdapter — FC 03 read-holding-registers decoding, full ADU + CRC16 validation (modbus_rtu.py).

ModbusTcpAdapter — FC 03 decoding with MBAP-header and length validation (modbus_tcp.py).

Shared Modbus logic — shared register-map decoding and validation helpers (modbus_common.py).

CorruptedPayloadError reason codes (extensible): TRUNCATED, BAD_CRC, WRONG_FC, INCOMPLETE_ARRAY, OUT_OF_RANGE, BAD_UNIT_ID, WRONG_PROTOCOL.

## 3.3 Edge Analytics PS (software aggregation)

The software half of the −75% volume KPI (REQ-001b). It consumes Frames and reduces a window of N frames into a single aggregated Frame (Decision 10: the canonical schema stays unchanged — the reducer value plus .min/.max suffixed measurements per quantity).

compute_stats — per-quantity window statistics: count, mean, rms, min, max, last (stats.py).

aggregate_window — N frames -> 1 Frame with reducer rms|mean|last plus <name>.min / <name>.max (aggregate.py).

dedupe — drop consecutive duplicate frames (aggregate.py).

WindowAggregator — stateful time-window aggregation, deterministic on collected_at (window.py).

## 3.4 Virtual test bench (50-meter simulator)

Hardware-free verification: a deterministic fleet of 50 simulated Modbus energy meters drives the adapters end-to-end (meters -> raw ADU -> Frame).

EnergyMeter — a single seeded, drifting meter (voltage/current/power/energy) (meters.py).

MeterFleet — a fleet of 50 meters with unique ids and unit_ids 1..50 (meters.py).

STANDARD_REGISTER_MAP — the shared 4-register layout (voltage_l1, current_l1, active_power, active_energy) (meters.py).

build_rtu_response / build_tcp_response — raw Modbus RTU (CRC16) and TCP (MBAP) FC 03 response builders (frame_builders.py).

Tests — fleet properties + southbound chain (RTU, TCP, 5000-frame load smoke test) (tests/).

## 3.5 Verification status (measured on 2026-10-05)

| Test suite | Result | Coverage |
| --- | --- | --- |
| Protocol Adapters (src/protocol-adapters) | 38 passed | adapters 100%; package 99% |
| Edge Analytics PS (src/edge-analytics/ps) | 19 passed | 100% |
| Virtual test bench (testbench) | 9 passed | n/a (integration/fleet) |
| Total | 66 passed | — |

The 1% uncovered in the Protocol Adapters package corresponds to two lines that are unreachable/defensive by design: the abstract ProtocolAdapter.parse() raise NotImplementedError, and an odd-byte-count guard in the shared decode helper. Both ModbusRtuAdapter and ModbusTcpAdapter themselves report 100% branch coverage. All numbers were reproduced by running the suites on the date above.

# 4. Prototype manual

## 4.1 Prerequisites

Python 3.10+ (verified on 3.14).

Linux shell; the project uses a local virtualenv.

No hardware required — the test bench is fully virtual.

## 4.2 Installation (one time)

cd projects/edgestream-gw/src/protocol-adapters

python3 -m venv .venv

.venv/bin/pip install -e ".[dev]"

The dev extra installs pydantic (runtime) plus pytest, hypothesis, pytest-cov and ruff (test tooling). The Edge Analytics PS and test-bench packages reuse this same virtualenv.

## 4.3 Running the test suites

# Protocol Adapters (38 tests)

cd projects/edgestream-gw/src/protocol-adapters

.venv/bin/pytest

# Edge Analytics PS (19 tests) — reuses the protocol-adapters venv

cd ../edge-analytics/ps

../../protocol-adapters/.venv/bin/python -m pytest

# Virtual test bench (9 tests)

cd ../../../../testbench

../src/protocol-adapters/.venv/bin/python -m pytest

# Coverage report

cd ../src/protocol-adapters

.venv/bin/pytest --cov=protocol_adapters --cov-report=term-missing

## 4.4 Using the Protocol Adapters (example)

The simulator and the adapters compose the southbound chain. This snippet parses one sample from every meter of the 50-meter fleet:

from meter_simulator import STANDARD_REGISTER_MAP, MeterFleet, build_rtu_response

from protocol_adapters import ModbusRtuAdapter, RawPayload

adapter = ModbusRtuAdapter(STANDARD_REGISTER_MAP)

fleet = MeterFleet(count=50, seed=0)

for meter in fleet:

payload = RawPayload(

protocol="modbus_rtu",

device_id=meter.device_id,

unit_id=meter.unit_id,

collected_at="2026-10-05T10:00:00Z",

data=build_rtu_response(meter.unit_id, meter.sample()),

)

frame = adapter.parse(payload)     # Frame, or CorruptedPayloadError

print(frame.model_dump_json(indent=2))

Fail-Fast behavior — malformed payloads are rejected with a structured error:

from protocol_adapters import CorruptedPayloadError, ModbusRtuAdapter, RawPayload

from meter_simulator import STANDARD_REGISTER_MAP

adapter = ModbusRtuAdapter(STANDARD_REGISTER_MAP)

bad = RawPayload(protocol="modbus_rtu", device_id="meter-001", unit_id=1,

collected_at="2026-10-05T10:00:00Z",

data=b"\x01\x03\x04\x00\x0a\x00\x14\x00")   # CRC missing

try:

adapter.parse(bad)

except CorruptedPayloadError as e:

print(e.reason_code)   # TRUNCATED

## 4.5 Using Edge Analytics PS (example)

from edge_analytics_ps import WindowAggregator, aggregate_window, dedupe

# frames: list[Frame] sharing the same source, ordered by collected_at

agg = aggregate_window(frames, reducer="rms")   # reducer: rms | mean | last

# Stateful time-window aggregation (deterministic on collected_at)

wa = WindowAggregator(window_seconds=60, reducer="rms")

for f in frames:

out = wa.add(f)        # None, or a Frame when a window closes

final = wa.flush()         # force-flush the last partial window

unique = dedupe(frames)    # drop consecutive duplicate frames

aggregate_window emits, per quantity, the reducer value plus <name>.min and <name>.max measurements, so N input frames collapse into exactly one output Frame — the mechanism behind the −75% volume KPI.

## 4.6 Running the examples outside pytest (PYTHONPATH)

The Edge Analytics PS and test-bench packages are imported via pytest bootstrap (conftest.py), so for a standalone script set PYTHONPATH explicitly:

cd projects/edgestream-gw

export PYTHONPATH="$PWD/src/protocol-adapters:$PWD/src/edge-analytics/ps:$PWD/testbench"

src/protocol-adapters/.venv/bin/python your_script.py

## 4.7 Known limits

Adapters — DLMS/COSEM is deferred to a dedicated specification phase (Decision 9); only Modbus RTU and TCP are implemented.

Edge Analytics — the −75% reduction is implemented but not yet end-to-end measured; the reduction-rate benchmark in performance/ is the next step.

Pipeline — Ingestion Core, Northbound Publishers, Store-and-Forward, Local Action Engine and the PL (FPGA) side are specified but not yet coded.

# 5. Requirements traceability summary

| Requirement | Component | Status |
| --- | --- | --- |
| REQ-001a — hardware data reduction (PL) | edge-analytics/pl/ | Specified (pending) |
| REQ-001b — software data reduction (PS) | edge-analytics/ps/ | Implemented (measurement pending) |
| REQ-002 — offline resilience | store-and-forward/ | Specified (pending) |
| REQ-003 — actuation < 50 ms | local-action-engine/ | Specified (pending) |
| REQ-004 — Fail-Fast (incl. 004.1–004.6) | protocol-adapters/ | Implemented & verified |
| REQ-005/006 — zero-config / OTA | deployment/ | Specified (pending) |
| REQ-007 — northbound interop | northbound-publishers/ | Specified (pending) |
| REQ-008 — embedded footprint | store-and-forward/ | Specified (pending) |

# 6. Roadmap / next steps

1. End-to-end reduction measurement: simulator -> adapters -> aggregate_window, then quantify the −75% volume KPI in performance/.

2. DlmsAdapter — a dedicated specification phase (framing HDLC vs COSEM-TCP, A-XDR decoding, OBIS mapping) per Decision 9.

3. Ingestion Core, Northbound Publishers, Store-and-Forward, Local Action Engine, then the PL (FPGA) partitioning.

4. Portfolio: Fog-Crypto Core and NetSentry-IoT detailed specifications.

# 7. Key risks (EdgeStream-GW)

Latency — PS/PL latency determinism — mitigated by hardware partitioning (Decision 4).

Resilience — data corruption during power outages — mitigated by SQLite ACID (Decision 3).

Compliance — Modbus/DLMS protocol compliance — mitigated by reusing proven stacks (Decision 1).

# Appendix A — Canonical Frame schema

Enforced with Pydantic v2 (extra="forbid", strict types, bounded ranges):

class Source(BaseModel):

model_config = ConfigDict(extra="forbid")

device_id: str

protocol: Literal["modbus_rtu", "modbus_tcp", "dlms_cosem"]

unit_id: int = Field(ge=0, le=247)

class Measurement(BaseModel):

model_config = ConfigDict(extra="forbid")

name: str

value: float

unit: str

quality: Literal["ok", "stale", "suspect"] = "ok"

class Frame(BaseModel):

model_config = ConfigDict(extra="forbid")

frame_id: UUID

source: Source

collected_at: str          # ISO-8601

measurements: list[Measurement] = Field(min_length=1)

JSON form:

{

"frame_id": "3f2f8c1e-...",

"source": { "device_id": "meter-001", "protocol": "modbus_rtu", "unit_id": 1 },

"collected_at": "2026-09-21T12:00:00Z",

"measurements": [ { "name": "voltage_l1", "value": 230.5, "unit": "V", "quality": "ok" } ]

}

Generated by makeTRM (reports/make_report.py) — Helveticore OÜ, Tallinn, Estonia. Report date: 2026-10-05.